# Resource: Metrics Framework and MCI Calculator (SDE_D3_M2_R3)

## Activity: Metrics and Dashboard Design

# Part 1: Metrics Selection Matrix

Instructions: Select the most relevant metrics for your case. Score each metric on relevance (1–5) and data availability (1–5). Prioritise metrics where both scores are high.

| # | Metric | Category | Unit | Relevance (1–5) | Data Available (1–5) | Selected? |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Material Circularity Indicator (MCI) | Circularity | 0–1 score |  |  |  |
| 2 | Recycled input rate | Input | % by mass |  |  |  |
| 3 | Recyclability rate | Output | % by mass |  |  |  |
| 4 | Product lifetime | Utility | Years / cycles |  |  |  |
| 5 | Waste generation rate | Waste | kg / unit |  |  |  |
| 6 | Energy from renewables | Energy | % of total |  |  |  |
| 7 | Water circularity | Water | % recirculated |  |  |  |
| 8 | Supply chain transparency | Governance | % traceable |  |  |  |
| 9 | DPP data completeness | Compliance | % of required fields |  |  |  |
| 10 | Carbon footprint per unit | Climate | kg CO2e |  |  |  |

# Part 2: MCI Calculation Worksheet

Instructions: Calculate the Material Circularity Indicator following the Ellen MacArthur Foundation methodology. Work through each step.

## Step 1: Input Data

| Parameter | Symbol | Value | Unit | Notes |
| --- | --- | --- | --- | --- |
| Mass of product | M |  | kg | Total mass of finished product |
| Mass of virgin feedstock | V |  | kg | Non-recycled, non-reused input |
| Mass of recycled/reused feedstock | Fr |  | kg | Input from recycled or reused sources |
| Fraction of feedstock from recycled sources | Fr/M |  | ratio | Calculated: Fr ÷ M |
| Mass collected for recycling at end of life | Cr |  | kg | Sent to recycling/reuse at EoL |
| Fraction collected for recycling | Cr/M |  | ratio | Calculated: Cr ÷ M |
| Efficiency of recycling (input) | Ef |  | ratio | Typical: 0.7–0.95 |
| Efficiency of recycling (output) | Ec |  | ratio | Typical: 0.7–0.95 |

## Step 2: Linear Flow Index (LFI)

LFI = (V + W) / (2M + (W_f − W_c)/2)

Where: V = virgin feedstock, W = unrecoverable waste, M = product mass

## Step 3: Utility Factor (F(X))

| Parameter | Value | Notes |
| --- | --- | --- |
| Actual lifetime (L) |  | Years of actual use |
| Industry average lifetime (L_av) |  | Average for product category |
| Actual use intensity (U) |  | Uses per time period |
| Industry average intensity (U_av) |  | Average for product category |
| Utility factor X = (L/L_av) × (U/U_av) |  | Calculated |
| F(X) = 0.9 / X |  | Applied to LFI |

## Step 4: MCI Result

| Calculation | Value |
| --- | --- |
| MCI = 1 − LFI × F(X) |  |
| MCI Score (0–1) |  |
| Interpretation | [What does this score mean for your product?] |
| Key improvement lever | [Which input has the biggest impact on MCI?] |

# Part 3: Dashboard Design Template

Instructions: Design a dashboard layout for presenting your selected metrics. Sketch the layout in the grid below, specifying visualisation type and data source for each element.

| Dashboard Element | Metric(s) Shown | Visualisation Type | Data Source | Update Frequency |
| --- | --- | --- | --- | --- |
| Top-left: KPI card | [Primary metric] | Single number + trend | [System] |  |
| Top-centre: KPI card | [Second metric] | Single number + trend | [System] |  |
| Top-right: KPI card | [Third metric] | Single number + trend | [System] |  |
| Middle-left: Chart | [Metric over time] | Line / bar chart | [System] |  |
| Middle-right: Chart | [Comparison metric] | Stacked bar / pie | [System] |  |
| Bottom: Table | [Detail data] | Sortable table | [System] |  |

Target Audience: [Who will use this dashboard? Executive, operational, regulatory?]

Drill-down Capability: [Can users click through to detail views?]

Alert Thresholds: [What triggers a warning or notification?]

This document is part of the Digital4Sustainability project, co-funded by the European Union.