Resource: Case Study Pack (CSS_D3_M1_R4)

GreenEdge Industries – Cybersecurity Risk Assessment Case Study

IMPORTANT: This document describes the use case context and three attack scenarios. Attack scenarios 1 and 2 are used for Face to Face lectures (Module 1, 2 and 3). The document also includes the case chapters.

These matirals are followed with an Excel file for each attack scenario with corrections CSS_D3_M1_R5 for scenario 1;  CSS_D3_M1_R6 for Scenario 2. and CSS_D3_M1_R7 for Scenario 3.

The teacher has a complete freedom to decide which elements to cover in face-to-face lessons and which to ask students to identify in class.

1. Scenario Briefing

GreenEdge Industries is a mid-sized manufacturing company operating across three production sites in Europe: Germany (Site 1), the Netherlands (Site 2), and Poland (Site 3), with approximately 800 employees. The organisation has adopted a strongly sustainability-oriented strategy, integrating environmental monitoring, ESG (Environmental, Social, and Governance) reporting, and operational optimisation into a unified digital ecosystem.

At the core of the sustainability programme is an ISO 14001-certified Environmental Management System (EMS), hosted on-premises at Site 1 in Germany. The system manages emissions tracking, waste, water usage, and energy consumption. A synchronisation module replicates data to Azure West Europe for advanced analytics and integration with external services.

Across the three sites, there is an IoT (Internet of Things) network with over 200 industrial sensors, including air-quality monitors, vibration sensors, and energy meters. Some sensors collect worker proximity data for safety purposes, introducing personal data processing. Data is transmitted via MQTT (Message Queuing Telemetry Transport) and CoAP (Constrained Application Protocol) to edge gateways and then securely forwarded to a cloud data lake.

The company uses a third-party ESG (Environmental, Social, and Governance) Software-as-a-Service (SaaS) platform to generate reports aligned with GRI (Global Reporting Initiative) and CSRD (Corporate Sustainability Reporting Directive). The provider is US-based with an EU data residency option, but risks related to international data transfers remain.

The IT (Information Technology) infrastructure includes Active Directory, Microsoft 365, VPN (Virtual Private Network) for remote access, and ERP and HR systems. HR data is integrated into the EMS to correlate safety events with employees, creating a direct link between operational and personal data.

At Site 2, legacy OT (Operational Technology) systems based on PLCs (Programmable Logic Controllers) use Modbus TCP without authentication. An OPC UA (Open Platform Communications Unified Architecture) gateway connects OT and IT, representing a critical convergence point.

GreenEdge must comply with GDPR, NIS2, ISO 14001, and ISO 27001. The integration of IT, OT, IoT, and cloud creates a complex attack surface where cyber risk, physical safety, environmental compliance, and data protection overlap.

2. Network Architecture Overview

Core Layers

OT/SCADA Layer (Site 2):

- PLCs for HVAC, water treatment, and waste management

- Modbus TCP protocol without authentication

- OPC UA gateway towards IT

IoT Edge Layer:

- Over 200 distributed sensors

- MQTT and CoAP protocols

- Edge gateways for data aggregation

IT Layer:

- Active Directory, ERP, HR

- Microsoft 365

- VPN and jump host

Cloud & SaaS Layer:

- Azure Data Lake (West Europe)

- ESG SaaS platform

- OTA (Over-The-Air) firmware portal

Integration & DMZ Layer:

- API gateway and OPC UA in DMZ (Demilitarised Zone)

- Firewalls between IT, OT, and cloud

Personal Data Processing Points

- Worker proximity sensors

- HR system

- Access logs and VPN

- CCTV (Closed-Circuit Television)

- HR–EMS integration

Trust Boundaries

- IT ↔ OT

- On-premises ↔ Cloud

- Organisation ↔ ESG SaaS

- Organisation ↔ OTA vendor

- Internal ↔ Remote access

- Physical perimeter

- EU ↔ Non-EU

## Attack Scenario 1 — Phishing ESG → Lateral Movement → Data/OT Pivot

An attacker launches a spear-phishing campaign targeting the ESG team using highly credible CSRD reporting templates. The email contains a link to a typosquatted domain replicating the ESG SaaS portal, prompting the user to enter Microsoft 365 credentials (T1566.002 – Spearphishing Link). These credentials are immediately used to access Office 365 and the VPN (T1078 – Valid Accounts), with Multi-Factor Authentication (MFA) bypassed through session token hijacking (T1550).

Once access is obtained, the attacker performs discovery on Azure AD and SharePoint to identify integrations with EMS and HR (T1087, T1082). If synchronisation scripts or stored credentials are exposed, privileges can be extended to on-premises systems (T1552 – Unsecured Credentials). From there, lateral movement is initiated via SMB and WinRM (T1021), targeting the EMS server in Germany.

The integration between HR and EMS enables linking assets to employees, increasing the value of exfiltrated data (T1005). In parallel, the attacker identifies the OPC UA gateway in the DMZ and uses it as a pivot point into the OT environment (T1046, T1021). Without strong segmentation, PLC enumeration becomes possible (T0888 – Modify Control Logic Discovery).

The attack concludes with double extortion: the exfiltration of personal data leading to a GDPR breach, and the manipulation of environmental parameters causing a direct operational impact (T1485, T1490). The likelihood remains high, as this scenario combines social engineering, credential reuse, and weak IT/OT integration. The Verizon DBIR continues to identify the human factor as a dominant driver in breaches.

## Attack Scenario 2 — Supply Chain IoT → Edge Compromise → OT Disruption

The attacker compromises the OTA (Over-The-Air) vendor used for IoT firmware updates (T1195 – Supply Chain Compromise). A backdoor is inserted into a signed but malicious update and distributed to edge gateways (T1608). After installation, the malware establishes command-and-control (C2) communication over MQTT, disguised as legitimate traffic (T1071.004 – Application Layer Protocol).

Edge gateways become persistent footholds (T1547) and begin collecting environmental data and local credentials, representing a lightweight variant of credential dumping on embedded devices (T1003). From the gateway, the attacker pivots into the local IT network, exploiting implicit trust between IoT and central systems. The attacker then identifies data flows to EMS and Azure Data Lake and manipulates ESG data (T1565 – Data Manipulation), creating a tangible risk of regulatory fraud under CSRD.

The next step targets OT via the OPC UA gateway (T0866 – Exploit Public-Facing Application in ICS context), and then reaches Modbus PLCs (T0831 – Manipulation of Control). The absence of authentication enables direct manipulation of the register (T0855). Impact may include environmental sabotage (e.g. falsified emissions data), production disruption, and physical safety risks.

The likelihood remains high due to increasing supply chain attacks and structural weaknesses in IoT/OT architectures. This scenario is driven both by a compromised update channel and implicit trust across edge, IT, and OT environments.

## Attack Scenario 3 — VPN Access → Stealth Persistence → Ransomware + Regulatory Collapse

VPN credentials of a technician are stolen via an infostealer (T1555) and later sold on underground marketplaces. The attacker uses this access to enter the internal network without triggering alerts (T1078), aided by plausible geolocation and absence of conditional access controls. A stealthy reconnaissance phase follows, lasting several days (T1083, T1018), remaining below detection thresholds (T1562).

The attacker then compromises Active Directory through delegation abuse and Kerberoasting (T1558.003), gaining elevated privileges. Once persistence is established, living-off-the-land tools such as PowerShell and PsExec are used for lateral movement (T1059, T1021). Backup systems are identified and neutralised (T1490).

In parallel, HR and EMS systems are accessed to prepare for personal data exfiltration (T1005, T1041). After weeks of undetected activity, ransomware is deployed simultaneously across IT systems and EMS servers (T1486), causing operational shutdown and loss of critical environmental data. Data leaks may also appear on dark web platforms, including employee data samples.

Failure to detect early-stage activity exposes the organisation to NIS2 violations in monitoring capabilities and GDPR non-compliance due to delayed breach notification. The likelihood is extremely high, as compromised credentials and prolonged dwell time remain among the most common incident patterns.