Resource: Use Case Study  for PBL activities (CSS_D3_M3_R8)

- AquaTerra Utilities – Sustainable Cybersecurity Case Study

INSTRUCTION: This document describes the use-case context and three attack scenarios for the PBL. The document also includes the case chapters.

Students will be asked to choose which attack scenario to develop (e.g., via a Doodle); once chosen, it will be the same for everyone.

This file is also followed with instructions for learners (CSS_D3_M3_R10)  and assessment criteria (CSS_D3_M3_R9) for trainers. The folder there are also:

- 1. Scenario Briefing

AquaTerra Utilities is a mid-sized regional water and wastewater utility cooperative serving approximately 1.2 million customers across three Nordic countries with around 1 100 employees. Operations are split across Site 1 in Oslo (Norway), where the headquarters and the main drinking-water treatment plant are located; Site 2 in Göteborg (Sweden), which hosts the wastewater treatment plant and a biogas recovery facility; and Site 3 in Aarhus (Denmark), which operates the pumping stations and the Advanced Metering Infrastructure (AMI) head-end for regional smart water meters.

Sustainability is central to AquaTerra's operating model. The cooperative is an essential entity under NIS2 (Network and Information Security Directive 2) and must comply with the EU Drinking Water Directive 2020/2184, the Urban Waste Water Treatment Directive, ISO 14001, ISO 27001, GDPR and the Corporate Sustainability Reporting Directive (CSRD). Core sustainability KPIs cover non-revenue water losses, leakage and energy per cubic metre treated, biogas yield from sewage sludge, and nutrient recovery rates. A dedicated ESG reporting SaaS is used to aggregate these figures and produce annual ESRS E3 (Water and Marine Resources) and E1 (Climate) disclosures.

At Site 1, a central SCADA (Supervisory Control and Data Acquisition) system supervises chemical dosing, ultraviolet disinfection, membrane filtration and reservoir levels. Process controllers use DNP3 (Distributed Network Protocol) and IEC 60870-5-104 over private MPLS links, while legacy pumping stations in Aarhus still rely on Modbus TCP without authentication. An OPC UA (Open Platform Communications Unified Architecture) tunnelling gateway in a dedicated DMZ connects OT historians to cloud-based analytics.

A fleet of roughly 6 500 water-quality IoT sensors (pH, turbidity, free-chlorine residual, flow rate) uses LoRaWAN (Long-Range Wide-Area Network) as the uplink, aggregated by a central Network Server hosted by a third-party operator. Approximately 420 000 smart water meters transmit consumption data via Wireless M-Bus (169 MHz) and NB-IoT (Narrowband Internet of Things) to the AMI head-end, which in turn feeds the SAP IS-U (Industry Solution for Utilities) billing platform.

The IT layer comprises Microsoft Active Directory, Microsoft 365, a customer self-service portal, a GIS (Geographic Information System) asset database, and an HR and shift-scheduling system. The AWS Frankfurt region hosts a data lake used for predictive maintenance and leak-detection machine-learning models. Personal data processing is substantial: customer name, address, account identifier, bank details, IBAN and half-hourly consumption profiles; employee shift rosters; and CCTV (Closed-Circuit Television) at unmanned pumping stations. The convergence of OT, IoT, IT and cloud creates a complex attack surface where cyber risk, public-health safety, environmental compliance and data protection all overlap.

- 2. Network Architecture Overview

Core Layers

OT/SCADA Layer (Sites 1, 2, 3):

- SCADA HMI servers and engineering workstations (AVEVA System Platform)

- PLCs (Siemens SIMATIC S7, Schneider Modicon M340) for dosing, UV, aeration and pumps

- DNP3 and IEC 60870-5-104 over private MPLS; legacy Modbus TCP at Aarhus pumping stations

- Biogas digester control system with custom SCADA tags feeding the EMS

IoT Edge Layer:

- ~6 500 LoRaWAN water-quality sensors (pH, turbidity, residual chlorine, flow)

- ~420 000 smart water meters using Wireless M-Bus 169 MHz and NB-IoT

- LoRaWAN Network Server (third-party operated) and AMI head-end concentrators

IT Layer:

- Active Directory, Microsoft 365, endpoint EDR

- SAP IS-U billing, GIS (ESRI), HR and shift-scheduling system

- Customer self-service portal in the DMZ

Cloud & SaaS Layer:

- AWS Frankfurt data lake, historian export and ML predictive-maintenance models

- ESG/CSRD reporting SaaS (EU-headquartered, with US-based sub-processor)

- Managed Microsoft 365 tenant with Entra ID

Integration & DMZ Layer:

- OPC UA tunnelling gateway between OT historian and cloud analytics

- Reverse proxy and API gateway for the customer portal and AMI-to-billing integration

- Jump host for OEM remote-maintenance sessions into SCADA

- Managed File Transfer (MFT) server exchanging regulatory files with national authorities

Personal Data Processing Points

- Smart water meter consumption profiles (half-hourly)

- Customer billing and payment records in SAP IS-U

- Customer portal accounts, DSAR intake and complaint tickets

- HR and shift-scheduling data for on-call engineers

- Access logs, VPN logs and OEM remote-maintenance session metadata

- CCTV footage at unmanned pumping stations and treatment sites

Trust Boundaries

- IT ↔ OT (MPLS and jump host)

- On-premises ↔ AWS Frankfurt data lake

- AquaTerra ↔ ESG/CSRD SaaS and its US sub-processor

- AquaTerra ↔ LoRaWAN Network Server (third-party operated)

- AquaTerra ↔ SCADA OEM (remote-maintenance vendor)

- Internal ↔ Remote customer portal users

- Physical perimeter at unmanned sites

- EU/EEA ↔ Non-EU data transfers (ESG sub-processor)

- Attack Scenario 1 — Billing Phishing → M365 Token Theft → SAP IS-U Exfiltration → IT Ransomware

An attacker launches a highly targeted spear-phishing campaign against the finance and billing team, impersonating a national water-regulator officer requesting reconciliation of tariff data. The lure is a Microsoft-branded sign-in page delivered via a malicious attachment with a QR-code redirect (T1566.001 – Spearphishing Attachment). The user authenticates through a reverse-proxy phishing kit that captures the session cookie (T1566.002 – Spearphishing Link) and bypasses multi-factor authentication by replaying the stolen token (T1550.004 – Web Session Cookie; T1078.004 – Valid Accounts: Cloud).

The adversary uses the hijacked identity to enumerate Entra ID groups, SharePoint sites and OneDrive content (T1087.004, T1082) and discovers a service-account credential stored in a finance runbook (T1552.001 – Credentials in Files). That account is reused against the on-premises SAP IS-U billing system (T1078), and internal reconnaissance continues over SMB, WinRM and MSSQL (T1021.002, T1021.006). Lateral movement reaches the customer self-service portal database and the CRM.

Approximately 420 000 half-hourly consumption profiles together with customer names, addresses, IBANs and contact details are staged and exfiltrated through a legitimate-looking OneDrive-for-Business share linked to a free-tier tenant (T1005 – Data from Local System; T1567.002 – Exfiltration to Cloud Storage). Immediately after exfiltration, the attacker deploys commodity ransomware across file servers, SAP application servers, the GIS database and the HR system (T1486 – Data Encrypted for Impact), after tampering with backup job schedules (T1490 – Inhibit System Recovery) and clearing Windows event logs (T1070.001). A double-extortion note demands payment to prevent publication of the billing data.

Likelihood is high: public-sector and utility billing teams are well-understood phishing targets, session-cookie theft now bypasses most push-MFA, and consumption data is directly monetisable. The dominant drivers are the human factor (Verizon DBIR), weak conditional-access policies, and credentials stored in documentation.

- Attack Scenario 2 — LoRaWAN Supply-Chain Compromise → Water-Quality Sensor Spoofing → Chemical Mis-dosing

The attacker compromises the third-party LoRaWAN Network Server operator by breaching a CI/CD pipeline used to publish firmware images for the concentrators and an update to the Network Server itself (T0862 – Supply Chain Compromise; T0817 – Drive-by Compromise on the vendor's developer portal). A trojanised release is signed with the vendor's legitimate code-signing key and distributed to AquaTerra's edge gateways and to the Network Server core, establishing covert command-and-control over MQTT (T0859 – Valid Accounts; T0885 – Commonly Used Port).

With root access on the Network Server, the adversary impersonates water-quality sensors and injects falsified pH, turbidity and residual-chlorine payloads into the uplink stream (T0856 – Spoof Reporting Message; T0855 – Unauthorized Command Message). The SCADA HMI at Site 1 trusts these readings and the automated dosing logic under-doses sodium hypochlorite while simultaneously over-dosing coagulant (T0836 – Modify Parameter; T0831 – Manipulation of Control). Cross-checks against laboratory samples are delayed because the attacker has also suppressed alarms by blocking specific uplink frames (T0814 – Denial of Service; T0839 – Module Firmware).

Concurrently, the attacker enumerates the AMI head-end and pivots towards the Aarhus pumping stations via the legacy Modbus TCP network, where unauthenticated function codes allow direct register writes against the pump PLCs (T0886 – Remote Services; T0836; T0855). The compound effect is a cyber-physical contamination incident: unsafe residual-chlorine levels in the distribution network, a boil-water advisory for an affected municipality, and an immediate notification obligation under the Drinking Water Directive, NIS2 Art. 23 and national public-health legislation.

Likelihood is rated high–to–medium: LoRaWAN supply chains remain immature, code-signing keys are often exposed in build systems, and unauthenticated Modbus remains pervasive. The driving factors are implicit trust across IoT/OT tiers and the absence of integrity checks on sensor payloads.

- Attack Scenario 3 — OEM Remote-Maintenance Foothold → Stealth Persistence → Historian Ransomware and Regulatory Collapse (used for PBL)

A SCADA OEM field engineer is infected by an infostealer after installing a cracked utility on a personal laptop that is later reused for a remote-maintenance session into AquaTerra (T1555 – Credentials from Password Stores; T1133 – External Remote Services). The stolen credentials, combined with a soft certificate exported from the jump host's keystore, grant the attacker access to the OT DMZ from a plausible geolocation and during an approved change window (T1078 – Valid Accounts).

For several weeks the adversary performs slow, low-and-slow reconnaissance (T1083 – File and Directory Discovery; T1018 – Remote System Discovery) while disabling endpoint logging on specific hosts (T1562.001 – Impair Defences: Disable or Modify Tools) and clearing selective log entries (T1070.001 – Clear Windows Event Logs). Kerberoasting of service accounts yields privileged access to the Active Directory forest (T1558.003), and lateral movement continues with PowerShell and PsExec (T1059.001, T1021.002). The historian (AVEVA PI), ESG data staging area and the SAP IS-U landscape are mapped, and the backup infrastructure is silently neutralised — scheduled jobs are disabled and immutable snapshot policies are quietly relaxed (T1490 – Inhibit System Recovery).

The adversary also exfiltrates staged personal data from HR and the customer portal (T1005; T1041 – Exfiltration Over C2 Channel) to prepare a secondary extortion lever. On a chosen night coinciding with a national holiday, ransomware is detonated simultaneously on IT, on the historian and on the ESG reporting landscape (T1486 – Data Encrypted for Impact). SCADA remains nominally operational, but the loss of the historian breaks CSRD ESRS E3 reporting continuity, leak-detection ML is blinded, and NIS2 Art. 23 significant-incident notification windows are triggered. Delayed breach notification under GDPR Arts. 33/34, combined with evidence that logging was disabled, creates a compounded regulatory-collapse scenario.

Likelihood is very high: compromised credentials and third-party remote-access paths remain the most common intrusion vectors in critical-infrastructure incidents, and dwell times measured in weeks are consistent with observed ransomware groups targeting utilities. The driving factors are trust in OEM maintenance paths, insufficient segmentation of the jump host, and fragile backup governance.